# Analysis of the effect in the electricity market equilibrium of a wind-power plant

Energy Markets - 2on Laboratory Assignment

Javier Muñoz Sáez (Author)

UPC

Catalonia- Spain

Javier.munoz.saez@upc.edu

Abstract—This report presents a study of a small energy distribution-generation-consumption network and examines the impact in market clearing and power flow of adding a wind power plant. GAMS (General Algebraic Modeling System) is used to find the market equilibrium for both scenarios.

Keywords: Energy Distribution; Energy Generation; Energy Consumption; Wind Power Plant Integration; Market Equilibrium; Day-Ahead Market; Renewable Energy; GAMS; Electricity Market Analysis; TCMA.

# Introduction

A simplified representation of the day-ahead electricity market of a fictitious country is used to analyze the optimization problem, modeling it as a TCMA (Transmission Constrained Multi-Period Auction Model).

The proposed fictitious market has 4 buses, 3 Demand points, 5 Generation points and 5 power lines is organized as seen in Fig 1.

Fig 1. Single line diagram of the studied network (WPP in blue)

The special case in which an extra WPP is added in the bus3 is highlighted in blue.

## Objectives of the study

The goal is to find the equilibrium points of the base case, which is the system without the wind power plant (WPP). After that, the WPP will be introduced at bus 3, and the equilibrium points identified by GAMS for both scenarios will be compared.

The key parameters for this comparison will be nodal prices, traded energy at each bus and time period, aggregated generation and demand functions, powerline congestions, economic transactions of producers and consumers, and merchandising surplus.

## Structure of the report

The report begins with a description of both model’s characteristics, with and without the WPP (Single-line diagram).

Next, point II explains in detail the data and how is translated into the TCMA mathematical model.

Following with point III, the results obtained by GAMS are presented. A MATLAB script is used to visualize the different market clearings for each bus and each time period.

Finally, the report ends with IV with brief conclusions and references.

# DEscription of the problem and mathematical model

## Presenting  model characteristics

All 5 Generation units are modeled following the data provided in Table1. Each Generator offers two different blocks (indexed as k) with its corresponding power in MW and price in €/Mwh. Limitations such as maximum/minimum powers and ramp up/down parameters are also provided; those define real market constraints.

Table 1, Base Model – Producers Characteristics

The extra WPP (wind power plant) characteristics is modeled following the data in Table2. This new power plant offers its limited energy (in comparison to the other generation plants) at 0 €/MWh, these 0 prices make its power very attractive and its included first in every auction. It does not have maximum or minimum power constraints, nor is it limited by any ramp-up or ramp-down limits.

Table 2, Generator 6, Windpower plant characteristics

The three demand points D1, D2, D3 (indexed by i) will be modeled using the parameters from Table 3. Similar to the generators in Table 1, these also offer multiple demand blocks, in this case three (indexed by k), each with its power in MW and associated price in €/MWh.

Table 3, Consumers Characteristics

Finally, the five transmission lines are modeled using the data from Table 4. These data complete this DC-OPF model with reactance xkl [p.u.], line capacity Smax [MW], and base power sb=100 [MW].

These lines will allow each bus to share excess power or excess demand with other buses

Table 4, Transmission lines Characteristics

## Presenting  mathematical ecuations (TCMPA transmission constrained multiperiod auction based equations)

The optimization objective is to maximize Social Welfare (SW) within every node (bus) where all powers (generated or demanded) are positive variables.

Equation 1, Social welfare maximization.

That SW is subject to the following rules Equation2

Equation 2, Matched generation and demand

The generation ramp up/down limits are modeled with Equation3.

Equation 3

The generation maximum/minimum limits translate to equation4.

Equation 4

As there is no energy storage modeled in our grid, the power generated must match the power consumed in every node Equation5 (+imports/exports).

Equation 5, Internodal market equilibrium

In order to model the power flow trough transmission lines, active power flow relates to grid frequency differences between buses Equation6 (frequency reference in bus1 = 0).

Equation 6, line power flows

Transmission line saturation is modeled with Equation7.

Equation 7, transmission line maximum power flow.

# Numerical results

GAMS was configured to find an optimal solution with 0.0 gap, this guarantees X*gams ≡ X*tcmpa.0

The interesting parameters from the GAMS found equilibrium are going to be:

- Market clearing (power and price) for each t

- Merchandizing surplus each t (power flows)

- Nodal clearings each node each t

- All different social welfare results (TCSW* SW* and NSW*) each t

## Results for the base case test T=1

There are no saturated Transmission lines so the whole market was able to clear at the same price of 35€/MW

Table 5, t=1 every transmission line power and price

Figure 2, Power flow between nodes in t=1

TCSW* was maximized to 22890.

Table 6, TCSW* for all T, t=1 highlighted in yellow

In bus1 some limitations were hit, for example G2 bottomed its Pmin of 28Mw so it was forced to sell unmatched power at a loss of 5€/MW.

G1 also hit its Pramp up limit of 100MW/t limiting its matched generation to 250Mw.

Figure 3, market clearing for bus1 t=1

Due to D1 offering to buy just 1MW almost all energy matched in bus1 was sold to other buses.

With bus2 having a very hungry D2, it would make sense for G3 and G4 to have all their generation matched, unluckily for those operators their bid was too expensive so energy imports from all other cheaper generators were in order.

Bus2 becomes an absolute energy importer in t=1 (Surplus generation -225)

Figure 4, market clearing for bus2 t=1

Both G3 and G4’s Rampdown limits were reached having to sell their energy also at 5€/MW loss.

Bus3 has no generation, and its demanded prices are very attractive for all other nodes, that’s why all its demand is matched with a virtual generation block.

The only exception are 15Mw that are using Bus3 as a buffer to channel generation from buses1 and 4 and routing it to bus2.

Figure 5, market clearing for bus3 at t=1

After all the optimization bus3 is setting the clearing price of the whole grid at 35€/Mw

Bus4 has only one generator so it makes sense all its energy bidding was matched and exported to other nodes.

Luckily for G5 operator the internodal market cleared the price higher than its offered price so its earning +5€ more per Mw.

Figure 6 ,market clearing for bus4 at t=1

## Results for the base case test T=2.

In a similar manner than T=1, transmission lines are not yet saturated so all nodes cleared to the same market price of 21€/Mw

Table 7, t=2 every transmission line power and price

The power flow remains the same as t=1 with slightly lower values.

Figure 7,market clearing for bus3 at t=2

Figure 8, Power flow between nodes in t=2

The overall demand plumbed and all the generation units were selling at a loss (surplus generation -1295) because they were all hitting their ramp-down and Pmin limitations, that’s why the low TCSW* =80008.

Table 8, TCSW* for all T, t=2 highlighted in yellow

Back to bus1, D1 is still demanding just a mere 1Mw, that makes G1 and G2 to be next exporters again.

G2 is still rock bottom at its Pmin 28Mw and now loosing even more money than before -9€/Mw.

Figure 9,market clearing for bus1 at t=2

G1 with its cheap energy offers capped its maximum Power of 300Mw and got a matching bid for all.

Things in bus2 are looking very similar, G3 and G4 bids are still very expensive, they still managed for some of their cheapest offers to be accepted. That makes bus2 to be still a net energy importer form every other cheaper energy sources.

Figure 10,market clearing for bus2 at t=2

Bus3, our demand hub is still marking the whole grid clearing price at 21€/Mw.

It is still used as a buffer for a small 11Mw going to bus2 
([1] Three agent transaction.)

Bus4 with its lonely G5, is still exporting all of its energy but unluckily the market cleared with a cheap energy price, G5’s operator is losing -9€/Mw in this situation.

G5 is hitting its Rampdown generation of -50Mw/t.

Figure 11,market clearing for bus4 at t=2

## Results for the base case test T=3.

Transmission lines are starting to be saturated, and the demand is suddenly increasing; this is taking away some of the power the demand unit D3 had over the whole market.

[1]Links which operate below their line-capacity may still have nodal price differences (congestion is not a line, it’s a network)

Table 9, t=3 every transmission line power and price, saturated line in red.

The power flow almost doubles since t=2, and now the line from bus1 to bus2 is saturated, with the line from bus1 to bus3 almost saturated aswel.

Figure 12 ,Power flow between nodes in t=3, saturated line in red

The market energy sharing between nodes is limited so energy price clearings are not the same between nodes anymore. It is starting to look more like a real market, TCSW* is higher because generation and demand sides are starting to reach compromises. (Previously the demand side was getting away with cheap energy prices and generators selling at loss).

Table 10, TCSW* for all T, t=3 highlighted in yellow

Back again to bus1, D1 still demanding neglible power, G1 and G2 are facing a sudden rise in demand from other buses.

The bad news for G1 is they reached their Prampup limit of 100Mw/t and they are already saturating all transmission lines, there is no space for any of the more expensive G2 power offers.

Figure 13,market clearing for bus1 at t=3

The good news for G1 operator is that they are earning a wooping +39€/Mw more than they were bidding for.

Even G2 with no bids matched yet is getting about +29€/Mw for their Pmin limited generation.

Figure 14,market clearing for bus2 at t=3

In bus2, G3 and G4’s expensive energy bids are starting to be matched.

They are both hitting their Prampup limits and earning some €/Mw

Bus2 is the one setting the most expensive clearing price of all, as there is a lot of demand now their expensive prices are being met.

Bus3, because of the congested line proving cheap energy from bus1, they are paying top euro price for some of the most expensive generation bids from alternative energy sources.

Figure 15,market clearing for bus34 at t=3

Generation operators in Bus4, they are the ones earning a lot of money now, G5 hits its Prampup of +50Mw and manages to sell all the energy that is able to produce at +55€/Mw to other nodes.

Figure 16,market clearing for bus4 at t=3

## Results for the base case test T=4.

Transmission lines are still saturated, but there is a slight shift in line flow, the saturated line is now the one from bus1 to bus3, the overall market demand is still high but not that much.

It is also worth noticing, merchandizing surplus in both buses going to bus2 is negative, this signals inefficiencies in the market, due to lower demand and Transmission line constraints.

Table 11, t=4 every transmission line power and price, saturated line in red.

The power flow panorama is very similar to the one in t=3, power flow directions hadn’t changed since the start of this simulation.

Figure 17, Power flow between nodes in t=4, saturated line in red

TCSW* is still high but starting to loosen due to drop in demand. Is also worth notice how the generation operators’ benefits are dropping (Surplus generation =7500, compared to 3x times in t=3)

Table 12, TCSW* for all T, t=4 highlighted in yellow

In Bus1, G1 and G2 scenario remains the same, G1 is matching most of its power bids and there is no transmission line resource left for G2 to include its power.

This time in bus1, G1 only allocated 270Mw of its 300Mw available.

Figure 18,market clearing for bus1 at t=4

In bus2, G3 and G4 bids are still being met but the demand didn’t change much so they are not hitting any of their Pramp limitations at the moment.

Figure 19,market clearing for bus2 at t=4

Bus3 D3 remains as the one paying the most expensive Mw.

The transmission line joining bus3 with its cheaper generation source bus1 is saturated so nodal market clearing is influenced by bus4 expensive imports.

Figure 20,market clearing for bus3 at t=4

Bus4 , also remains similar, the optimizacion TCMPA is still dealing a good price for G4, now earning a 42€/Mw more than the price they were bidding.

Figure 21,market clearing for bus4 at t=4

## Results for the base case test T=5.

Not much to comment, same line saturated than in t=4.

Same generation and demand volume than in t=4, and slight increase in Transmission line usage.

Same market inefficiencies signaled by negative merch surplus in bus2.

Table 13, t=5 every transmission line power and price, saturated line in red.

Bus1 is still saturating its Transmission lines trying to export all its energy.

Table 14,Power flow between nodes in t=5, saturated line in red

TCSW* is very similar to previous cases, dropping a bit due to slightly higher usage of Transmission lines.

Table 15, TCSW* for all T, t=5 highlighted in yellow

Practically all generation bids from G1 were matched in bus1 this time, this means G1 is almost hitting its Pmax again and both L1-2 and L1-3 are saturated again.

G2 still remains out of the bidding providing its usual Pmin=28Mw.

Figure 22,market clearing for bus1 at t=5

Bus2 clears its market fulfilling all demand from D2, matching the cheapest biddings from local generators G3 G4, importing everything else from bus4 and bus3 (proxy of bus1)

Figure 23,market clearing for bus2 at t=5

Bus3, as usual paying the most from all energy imports, as the market’s transmission lines are oversaturated.
Clearing the market far away from the optimal point, this indicates heavy inefficiencies in the market energy allocation.

Figure 24,market clearing for bus3 at t=5

In bus4 G5 as usual matching its only bid, this time the market inefficiency is leaning heavily against the consumers.

Figure 25,market clearing for bus4 at t=5

## Results and Analysis for the case with the inclusion of the Wind turbine

Launching the same optimization problem but now adding a low power renewable generator G6 located in D3.

By comparing against the base case (no WPP), the following graphs will help us identify in who’s interest is the addition of this new Wind power plant.

There is an overall increase in the utilization of all not saturated lines.

The addition of the WPP didn’t help clear any of the bottlenecks (congestion taxes) , in this case the TSO would not be interested in the WPP unless higher power turbines are installed.

The addition of WPP served only to cover D3 huge power demand, this means all clean energy produced by the WPP was spent localy in bus3.

It would be in the Demand side’s interest to install this WPP , specially D3, and indirectly the other demand actors .

Leaning on the side of the generators, G1 would be a bit interested in the contruction of the new WPP, but G3 on the other hand will loose a big chunk of its matched power if the new WPP is put in place.

The impact of the WPP is very low when the transmission lines are not congested due to having an order of magnitude lower power offer than the other generator counterparts, but it is big enough to clear a couple of bottlenecks in t4 and t5, specially affecting bus1 clearing price.

What about nodal SW* our optimization parameter.

Bus3 is greatly benefited since start by having the local WPP, it is not the case of every other bus when Transmission line saturation happens.

# Conclusions

As an initial conclusion, it is always advantageous for a community of consumers without local energy generation to be able to install a wind power plant (WPP) to supply at least part of their local consumption.

Studying the global impact of such a WPP on the network of 4 buses, it makes sense to construct it to alleviate an already congested grid. However, it would be necessary to build a WPP with greater capacity to effectively relieve the congestion on the L1-3 line.

The results indicate that the installation of the WPP is in the interest of D3. However, there are many generating actors who benefit from the line congestion, and thus it is not in their economic interest to support the construction of this new plant. If these generating actors had a say in the decision, it would make sense for them to oppose the construction of the WPP.

As a detail, a simulation of a WPP with 30 times the installed capacity was tested, and even with this power, the line congestion was not alleviated. Therefore, it would be more effective to opt for the installation of more distributed generation plants, particularly at bus 3 and bus 2, which are the main energy consumers.

# Reference

“Nodal prices and transmission rights: A critical appraisal” by Shmuel S. Orena, Pablo T. Spillera, Pravin Varaiyaa and Felix Wua (The Electricity Journal, Volume 8, Issue 3, April 1995, Pages 24–3